Requirements Document

SNMP Proxy Requirements

Revision 3.0

Team 6

Contracted by Printronix

ICS 125

Summer, 2002

Professor Hadar Ziv

Information in this document is subject to change without notice.

Permission is hereby granted to reprint or retransmit this document unaltered, in original, printed or photocopied format as required.

© 2002 Printronix.  All Rights Reserved Worldwide.

Java™ and all Java-based trademarks and logos are trademarks or registered trademarks of Sun Microsystems, Inc. in the United States and other countries.

Table of Contents

Introduction	4

DOCUMENT VERSION HISTORY	5

Project Schedule	6

Required Tasks	6

Resource Table	7

Required Deliverables	8

Project Risks	8

Project Resources	10

Project Memebers	10

Required Hardware and Software	10

Staff Organization	11

Tracking and Control Mechanisms	11

Requirements Specification	12

Overview	12

Enviornmental Characteristics	12

Domain Specific Rules	12

UI model	13

Systems Analysis	13

Use Cases	14

Combined Use Case Diagram	14

Request General Proxy Information	15

Manage SNMP Access Control	17

Issue Device Commands	20

Configure Device Boot Settings	23

Configure Printer Trap Conditions and SNMP Alert	26

Configure Printer Trap Conditions and SMTP (Email) Alert	29

Non-Functional Constraints	32

Efficiency	32

Process	32

External	32

Lifecycle Considerations	32

Acceptance Requirements	33

Unit Test Plan	33

Integration Test Plan	35

System Test Plan	43

Glossary	45

Introduction

The current project under development by Team 6 is a SNMP proxy for Printronix.  This project aims to enhance the networking management functionality of their printing devices.  The proxy will greatly improve management flexibility of Printronix printer devices since administrators will be able to configure those devices remotely using any standard SNMP management console.

In the required system, clients at multiple SNMP management consoles communicate with the SNMP proxy via SNMP protocol.  The SNMP proxy processes and translates client request into XML messages in order to communicate with a non-SNMP-based device management server, which can then configure various non-SNMP-enabled Printronix printer devices.  The SNMP proxy will receive responses back from the device management server in the form of XML messages that it will deliver via SNMP to the appropriate SNMP management console.

The management consoles intended to be using the SNMP proxy system are based on the SNMP version 1 protocol.  The device management server communicates with SNMP proxy via XML.  The proxy will allow communication between two services that would otherwise not be able to do so.  Adhering to SNMP, an industry standard (though somewhat arcane) will result in the SNMP proxy being highly compatible with existing management systems.  Since the SNMP proxy will be an internal system, it will not include a user interface.

This document contains the following sections:

- Introduction –  the introduction gives an abstract overview of the system to be implemented

- Project Schedule

- Required Tasks – list of tasks that must be completed in order to complete your project

- Resource Table - indicates how each team member will be applied throughout the weeks of the project

- Required Deliverables – dates deliverables are due

- Project Risks – identifies risks that are unique to this project

- Project Resources - List the people on your team and describe their personal and technical strengths,  as well as the hardware, software, and other equipment that will be required to complete the project

- Staff Organization – outlines the team structure and member responsibilities

- Tracking and Control Mechanisms – lists how deliverables are controlled and how versions are tracked

- Requirements Specification

- Overview of System Requirements - provides a brief discussion of basic needs and proposed usage.

- Environment Characteristics - describes hardware, software, and users of the system.

- Use-cases – gives a detailed description of all the use-cases and their scenarios, with pre- and post-conditions

- Non-functional Constraints - lists other relevant information that pertains to the software

- Domain-Specific Rules - specifies all the rules related to the domain to which the software will be applied that must be adhered to.

- UI model – identifies status of user interface. MISSING SECTIONS

- Lifecycle Considerations – outlines the lifecycle this project will follow

- Acceptance Requirements – describes the minimal requirements for system acceptance by the client

- System Test Plan - a test plan capable of demonstrating minimal functionality of all system elements

- Glossary - defines all technical terms used in the specifications

Document Version History

Revision 1.0

- Initial use case analysis completed.

Revision 2.0

- Extensive requirements gathering completed, representing a more complete picture of the system.

- Initial use cases modified to reflect new findings.

- Glossary expanded

- Initial outlines completed for integration test plan.

Revision 2.1

- Minor modification of use cases resulting from a refined understanding of the required protocol sequence.

- Grammatical errors corrected.

- Modification of Environmental Characteristics section.

- Addition of a System Analysis section.

Revision 3.0

- Use cases modified to become slightly more focused as a result of sequence analysis (see Design Document  revision 1.0)

- Unit, integration and system test plans outlined and refined under the System Test Plan.  Further revisions of the unit test plans are expected during final design phases.

- Modifications to the Tracking and Control Mechanisms section.

Minor modifications to all sections.

Project Schedule

Required Tasks

| Task | Start Date | Completion Date |
| --- | --- | --- |
| Research SNMP Protocol commands | July 5 | August 6 |
| Research XML syntax and usage | July 5 | August 6 |
| Develop the project website | July 6 | July 9 |
| Develop requirements iteration 1 | July 8 | July 11 |
| Meet with project client to discuss requirements | July 8 | July 15 |
| Maintain a team log | July 9 | pending |
| Receive Device Management Server (DMS) from Printronix | July 9 | NA |
| Develop requirements iteration 2 | July 11 | July 17 |
| Develop test plan iteration 1 | July 13 | July 18 |
| Research available Java SNMP libraries | July 15 | August 6 |
| Receive preliminary DMS XML specification from Printronix | July 16 | July 16 |
| Develop test plan iteration 2 | July 18 | July 25 |
| Develop design iteration 1 | July 18 | July 25 |
| Establish simulation environment | July 25 | July 30 |
| Develop design iteration 2 | July 25 | August 6 |
| Develop code iteration 1 | July 25 | August 6 |
| Receive final DMS XML specification from Printronix | August 6 | August 6 |
| Develop code iteration 2 | August 6 | August 27 |
| Simulate Printronix LAN environment | August 20 | August 27 |
| Develop final deliverables | August 25 | August 27 |
| Present project to clients | August 27 | August 30 |

Resource Table

| Member | Iteration | Assigned Tasks |
| --- | --- | --- |
| Anaya, Raymundo | Requirements | - Research project risks - Introduction - Lifecycle Consideration - Use Case contribution |
|  | Design | - Introduction - Design Specification |
|  | Test | - (pending) |
|  | Implementation | - (pending) |
|  | (General) | - Maintain team log - Research XML |
| King, Christopher | Requirements | - Research project risks - Introduction - Use Cases - Version updating |
|  | Design | - Project Plan - Sequence analysis - Design Specification |
|  | Test | - (pending) |
|  | Implementation | - (pending) |
|  | (General) | - Maintain team web site |
| Liao, Allen | Requirements | - Introduction - Project Schedule - Project Resources - Use Cases contribution - Glossary |
|  | Design | - Design Specification - System Test Plan - Unit Test Plan |
|  | Test | - (pending) |
|  | Implementation | - (pending) |
|  | (General) | - Research SNMP tools and libraries |
| Hu, Susan | Requirements | - Use Case diagrams - Preliminary Introduction |
|  | Design | - Design Specification - Tracking and Control Mechanisms - SNMP research |
|  | Test | - (pending) |
|  | Implementation | - (pending) |
|  | (General) | - Research SNMP |
| Printronix |  | - Provide SNMP MIB table - Provide Printronix Output Management Enterprise Server XML commands |

Required Deliverables

| Deliverable Item | Due Date |
| --- | --- |
| Team Webpage | Tue, July 9 |
| Requirements Iteration 1 | Thu, July 11 |
| Requirements- Iteration 2 / Test Plan iteration 1 | Thu, July 18 |
| Test Plan Iteration2/ Design Iteration1 | Thu July 25 |
| Design Iteration2/ Code -Iteration1 | Tue August 6 |
| Code-Iteration2 (Final)+ all final deliverables | Tue August 27 |
| Presentation of project to Printronix | Thu/Fri August 29/30 |

Project Risks

Limited Schedule

| Description | The schedule according to which the SNMP Proxy is being developed is extremely aggressive |
| --- | --- |
| Consequences | May result in a product that does not satisfy the needs of Printronix. |
| Likeliness | Somewhat Likely |
| Avoidance | Adhere to project time tables |
| Monitor | Using team logs and noting completion dates |

Unfamiliarity with required protocols

| Description | Project members may not become sufficiently familiar with SNMP protocol and XML within given time constraints. |
| --- | --- |
| Consequences | The product may be of inferior quality and/or may not function properly limiting its use to the customer. |
| Likeliness | Likely |
| Avoidance | Gather all available resources concerning required protocols for review and study. |
| Monitor | Discussion between team members concerning their familiarity with required protocols during different phases of development. |

System Timing Issues

| Description | The SNMP Proxy is subject to various timing constraints which may lead to timeouts if the Proxy does not respond within the allotted amount of time. |
| --- | --- |
| Consequences | If the Proxy times out frequently, its usefulness will be significantly limited. |
| Likeliness | Very Likely |
| Avoidance | The SNMP Proxy will be designed with timing in mind by monitoring potential timeouts and sending “keep alive” messages to prevent connections from timing out. |
| Monitor | During testing phases, system times will be monitored for timeouts to test system responses. |

Security Risks

| Risk Identification | The SNMP proxy can become subject to security issues because of unauthorized queries. |
| --- | --- |
| Description | Since all of Printronix’s computers are running SNMP and are listening on the same port, the can all be queried and important system information can be retrieved by the Device Management Console. |
| Consequences | Users of the Device Management Console may be able to retrieve sensitive information from Printronix computers. |
| Likeliness | Very Likely |
| Avoidance | The proxy will be configurable to listen on an independent port. |
| Monitor | It will be made sure that information cannot be gathered from outside sources during system integration and testing phases. |

Hardware Limitations

| Description | Hardware simulations may contain inaccuracies (i.e. improper simulations of concurrent open sockets, network traffic etc.) allowing implementations problems to go unnoticed until final testing. |
| --- | --- |
| Consequences | Significant bugs may be discovered to late in the project schedule to make necessary corrections before deliverable due dates. |
| Likeliness | Very Likely |
| Avoidance | Careful consideration must be given the simulation in addition testing should also be done on the actual target hardware whenever possible. |
| Monitor | Testing will be performed on the actual target hardware to discover potential problems during simulation testing. |

Client Deliverables

| Description | Certain deliverables that are required from the client including the Device Management Server XML specifications and the Device Management Server itself may not be received within the scheduled period. |
| --- | --- |
| Consequences | The project may be delayed during different phases of development. |
| Likeliness | Not Likely |
| Avoidance | Inform Printronix in advance of upcoming scheduling issues and dates. |
| Monitor | Close contact with Printronix |

Project Resources

Project Memebers

| ##### Name | ###### Skills |
| --- | --- |
| Ray Anaya | - Proficient in Java - Knowledgeable in XML - Experienced in writing requirements documents, design documents, and test plans - Proficient in UML - Experience in designing Use Cases |
| Susan Hu | - Proficient in Java - Experienced in writing design documents and test plans - Proficient in UML - Experience in designing Use Cases |
| Chris King | - Proficient in Java - Knowledgeable in XML - Experienced documenting requirements and developing software designs - Proficient in UML - Experience in designing Use Cases - Experience creating web pages |
| Allen Liao | - Proficient in Java - Experienced in writing requirements documents, design documents, and test plans - Proficient in UML - Experience in designing Use Cases |

Required Hardware and Software

- Networked computers (at least 2)

- Simulation of the Printronix Device Management Server

- Java compiler

- Java SNMP libraries

Staff Organization

The four members of Team 6 are responsible for each contributing equally to the development and quality of the project.  The central point of communication will be conducted through the project web site at URL:  http://team6project.tripod.com.  The web site contains a discussion board in which both clients’ and project members’ discussions are archived for viewing.  The site will also act as a clearing house for all project information and status.

Chris King is responsible for posting the reports via the web site, so all project whereabouts are directed to him.  However, team members still report all whereabouts to all other team members.

Managers are rotated on a development phase basis.  A manager’s duty includes directly contacting the customer when be as well as communicating with the professor.  Clerical tasks are rotated on a development basis as well, which includes the recording of team logs.

x = manager	o = clerical

| Name | Requirements | Design | Implementation | Test |
| --- | --- | --- | --- | --- |
| Ray Anaya | x |  |  | o |
| Susan Hu |  | x | o |  |
| Chris King |  | o | x |  |
| Allen Liao | o |  |  | x |

Tracking and Control Mechanisms

The strict schedule is to be followed to the exact date (refer to Project Schedule).  As delays cannot be accurately predicted, the current goal of the project will always be presented on the web site.

As each document will have separate contributions from each team member, each version that a member modifies is stored on the web space in a personal folder with the version date.  Modifications are then combined into a publishable version by the team manager.  Each member is responsible for his/her assigned sections of the documents.  Upon the deliverable date and time, the combined versions are published on the team website.

Subsequent modifications to a published document are republished under a new version number.  The most up-to-date publishable candidate or modified delivered document will always be available on the web site.

During the Implementation phase, each module is archived upon successful implementation.  This ensures that all modifications to a successfully implemented module are tracked as integration efforts are conducted.

Requirements Specification

Overview

The required system is a SNMP proxy for Printronix in order to enhance the networking functionality of their printing devices.  In the required system, clients at multiple SNMP management consoles communicate with the SNMP proxy via SNMP protocol.  The SNMP proxy processes and translates client request into XML messages in order to communicate with a non-SNMP-based device management server, which can then configure various non-SNMP-enabled Printronix printer devices.  The SNMP proxy will also receive responses back from the device management server in the form of XML messages that it will deliver via SNMP to the appropriate SNMP management console.

Enviornmental Characteristics

The SNMP Proxy will be run on a network consisting of a server running the Printronix Output Management Enterprise Server (POMES) that is connected to various Printronix devices over the network.  These devices will essentially be Printronix printers.  Device Management Consoles (DMCs) will also be running on the network, possibly listing on the same port as the SNMP Proxy.

The implementation of the SNMP Proxy must conform to the SNMP v1 standard specified by RFC 1156 to communicate with DMCs.  In addition, the SNMP Proxy must also handle the XML command protocol specified by XMLCommands.doc (an internal Printronix specification not included in this document) in order to communicate with the POMES.

The SNMP Proxy will be implemented in JAVA making the system fairly platform independent.  In general, it will be executed on Microsoft Windows NT Server platforms.

During development phases, a simulation of the POMES server will be used to simulate communications with Printronix devices on a basic network lacking Printronix hardware.

Users of the system are system administrators who manage their networks with DMCs via SNMP protocol who wish to manage Printronix devices.

Domain Specific Rules

Timing restrictions must be taken into consideration for the SNMP Proxy.  There is a timeout period specified by SNMP v1 that all SNMP message replies must be received by the management consoles in or else a time out error occurs.  As a result an SNMP protocol message must be replied to with a “keep alive” message (i.e. a text stating “waiting for server”) while the SNMP proxy is communicating with the POME to prevent timeouts with the management consoles.

UI model

The SNMP Proxy is an internal system that has no direct contact with end users of the system.  Instead, it interacts with two independent systems over a network and manages communications between them.

Systems Analysis

The following diagram outlines the basic layout of the system.  Note that the implementations of the device server and SNMP Proxy are in Java, making the underlying hardware irrelevant due to Java’s virtual machine.

SNMP Messages

XML Messages

SNMP Messages

Use Cases

Combined Use Case Diagram

Request General Proxy Information

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Request General Proxy Information |
| Status |  | Under development |
| Author |  | Team 6 |
| Purpose |  | To provide general SNMP Proxy information to SNMP Device Management Console users. |
| Overview |  | Users using an SNMP Device Management Console (DMC) will be able to request general information from the proxy including the proxy name and version, technical support information as well as Printronix contact information. The DMC sends a SNMP message containing an OID of the desired information object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the requested object to the correct DMC (see Notes 2) using a stored IP Address. The use case will fail if either the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | N/A |
| Precondition |  | - SNMP Proxy listening on specified port for a SNMP UDP packets from DMCs - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | Information requested from the SNMP Proxy is received by the correct DMC that requested the information. |
|  | Failure | No response received by the DMC that requested the information from the SNMP Proxy within the timeout period or an error has occurred (i.e. as a result of an invalid OID) that was reported and received by the correct DMC (see Note 5) from the SNMP Proxy that indicates the reason for the failure. |
| Related Use Cases |  | N/A |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - The correct DMC is the DMC that initially messaged the SNMP for that response. This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: General-1, General-1b, General-1c, General Failure-1 |
| Basic Course |  | - DMC sends a SNMP “get” message using one of the OIDs of the “general” MIB subtree (see Notes 1) to the SNMP Proxy. The OID corresponds to the proxy name and version, technical support information as well as Printronix contact information depending on the actual OID address. - SNMP Proxy receives the “get” SNMP message. - SNMP Proxy logs the IP address and Port of the DMC making the request. - SNMP Proxy sends an SNMP response object containing the requested SNMP Proxy information using the logged IP Address. - DMC receives the requested proxySNMP response object. |
| Successful alternate course |  | For Step 2 A2.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A2.2 The DMC may repeat Step 1. (See Notes 3) |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy For Step 6: F 6.1 The DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. |
| Open Issues |  | - Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? |

Manage SNMP Access Control

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Manage SNMP Access Control |
| Status |  | Under development |
| Author |  | Team 6 |
| Purpose |  | To provide functions to control access to SNMP functions through the SNMP Proxy. The primary control setting is the Community Name (see Notes 3) |
| Overview |  | Users using an SNMP Device Management Console (DMC) will be able to manage SNMP access to devices through the use of an SNMP community name (used as a weak password protection scheme). The DMC first sends a message to the SNMP Proxy indicating which device it would like to manage. The DMC then sends a SNMP message containing an OID (see Note 1) of the desired community object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the requested object to the correct DMC (see Notes 5) using a stored IP Address. The use case will fail if the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | Printronix Object Management Enterprise Server (POMES) |
| Precondition |  | - SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. - SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | An SNMP response object is received by the correct DMC (see Notes 5) from the SNMP Proxy that indicates that the command was successful. |
|  | Failure | No response received by the DMC that requested the information from the SNMP Proxy within the timeout period or an error has occurred (i.e. as a result of an invalid OID) that was reported and received by the correct DMC (see Note 5) from the SNMP Proxy that indicates the reason for the failure. |
| Related Use Cases |  | N/A |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - See XMLCommands.doc (not included in this document) - The POMES server maintains the Community name for the SNMP proxy to use. - A User is expected to instruct the DMC to repeat an SNMP request upon receiving a “waiting for server” message. This is necessary because communication with POMES will presumable take longer then the DMC timeout period and because the SNMP Proxy cannot send a response message to the DMC with out the DMC explicitly requesting it. - This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: Manage-1, Manage-1a, Manage-1b, General Failure-1, General Failure-2 |
| Basic Course |  | - DMC sends a SNMP “set” message (see Notes 1) containing the IP address of the device the that DMC user wishes to manage. - SNMP Proxy receives the SNMP “set” message - SNMP Proxy logs the IP address and Port of the DMC making the request (see Note 5) as well as the IP of the device the DMC user wishes to manage. - SNMP Proxy sends an SNMP response message to the DMC indicating “managed IP set”. - DMC receives the response message. - DMC sends a SNMP message using one of the OIDs (see Notes 1) that corresponds to the SNMP community name object for the managed device that will either be “get” or “set” depending on the actual OID address. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC repeats SNMP request sent in step 6 (via DMC User response to message sent in Step 11, see Notes 4) . - SNMP Proxy receives the repeated SNMP message. - SNMP Proxy sends an SNMP response to the DMC containing the corresponding response object that the DMC initially requested in Step 6. - DMC receives the requested response object. |
| Successful alternate course |  | For Step 5 A5.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A5.2 The DMC may repeat Step 1. (See Notes 6) For Step 15: A 15.1 SNMP Proxy receives the repeated SNMP message from the DMC but the SNMP Proxy has not received the required XML stream from the POMES (i.e. Step 15 did not occur). A 15.2 SNMP Proxy sends an SNMP message to the DMC indicating “waiting for server”. The DMC may repeat Step 10. |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy despite repeated attempts For Step 5: F 5.1 DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. For Step 8: F 8.1 SNMP Proxy connection to POEMS fails F 8.2 SNMP responds to DMC request with “server connection error” |
| Open Issues |  | Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? |

Issue Device Commands

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Issue Proxy Commands |
| Status |  | Under development |
| Author |  | Team 6 |
| Purpose |  | To provide the ability to issue device commands via the SNMP proxy to SNMP console users. |
| Overview |  | Users using an SNMP Management Console will be able to issue a command to reset a Printronix device. A list of variables (currently unspecified) should also reset the device. Resetting a device will result in recent configuration modifications made by the user via the SNMP proxy to take effect. The DMC first sends a message to the SNMP Proxy indicating which device it would like to manage. The DMC then sends a SNMP message containing an OID (see Note 1) of the desired command object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the response object to the correct DMC (see Notes 5) using a stored IP Address. The use case will fail if either the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | Printronix Object Management Enterprise Server (POMES) |
| Precondition |  | - SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. - SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | An SNMP response object is received by the correct DMC (see Notes 5) from the SNMP Proxy that indicates that the command was successful. |
|  | Failure | No response received by the correct DMC from the SNMP Proxy within the timeout period or an error has occurred (i.e. as a result of an invalid OID) that was reported and received by the correct DMC (see Note 5) from the SNMP Proxy that indicates the reason for the failure. |
| Related Use Cases |  | N/A |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - See XMLCommands.doc (not included in this document) - The POMES server maintains the Community name for the SNMP proxy to use. - A User is expected to instruct the DMC to repeat an SNMP request upon receiving a “waiting for server” message. This is necessary because communication with POMES will presumable take longer then the DMC timeout period and because the SNMP Proxy cannot send a response message to the DMC with out the DMC explicitly requesting it. - The correct DMC is the DMC that initially messaged the SNMP for that response. This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: Commands-1, Commands-1a, Commands-1b, General Failure-1, General Failure-2 |
| Basic Course |  | - DMC sends a SNMP “set” message (see Notes 1) containing the IP address of the device the that DMC user wishes to manage. - SNMP Proxy receives the SNMP “set” message - SNMP Proxy logs the IP address and Port of the DMC making the request (see Note 5) as well as the IP of the device the DMC user wishes to manage. - SNMP Proxy sends an SNMP response message to the DMC indicating “managed IP set”. - DMC receives the response message. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP command object for the managed device that will instruct the device to reset. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC repeats SNMP request sent in Step 6 (via DMC User response to message sent in Step 11, see Notes 4) . - SNMP Proxy receives the repeated SNMP message. - SNMP Proxy sends an SNMP response to the DMC containing the corresponding response object that the DMC initially requested in Step 6. - DMC receives the requested response object. |
| Successful alternate course |  | For Step 5 A5.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A5.2 The DMC may repeat Step 1. (See Notes 6) For Step 15: A 15.1 SNMP Proxy receives the repeated SNMP message from the DMC but the SNMP Proxy has not received the required XML stream from the POMES (i.e. Step 13 did not occur). A 15.2 SNMP Proxy sends an SNMP message to the DMC indicating “waiting for server”. The DMC may repeat Step 14. |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy despite repeated attempts For Step 5: F 5.1 DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. For Step 8: F 8.1 SNMP Proxy connection to POEMS fails F 8.2 SNMP responds to DMC request with “server connection error” |
| Open Issues |  | Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? What list of variables also causes a device reset? |

Configure Device Boot Settings

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Configure Device Boot Settings |
| Status |  | Under development, subject to design changes |
| Author |  | Team 6 |
| Purpose |  | To provide functions to configure boot settings for Printronix devices through the SNMP proxy. |
| Overview |  | Users using an SNMP Management Console will be able to configure which boot protocol will be used to determine a device’s IP address as well as which boot server will be used by the device. The DMC first sends a message to the SNMP Proxy indicating which device it would like to manage. The DMC then sends a SNMP message containing an OID (see Note 1) of the desired boot data object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the response object to the correct DMC (see Notes 5) using a stored IP Address. The use case will fail if either the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | Printronix Object Management Enterprise Server (POMES) |
| Precondition |  | - SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. - SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | An SNMP response object is received by the correct DMC (see Notes 5) from the SNMP Proxy that indicates that the boot protocal configuration was successful was successful. |
|  | Failure | No response received by the DMC that requested the information from the SNMP Proxy within the timeout period or an error has occurred (i.e. as a result of an invalid OID) that was reported and received by the correct DMC (see Note 5) from the SNMP Proxy that indicates the reason for the failure. |
| Related Use Cases |  | N/A |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - See XMLCommands.doc (not included in this document) - The POMES server maintains the Community name for the SNMP proxy to use. - A User is expected to instruct the DMC to repeat an SNMP request upon receiving a “waiting for server” message. This is necessary because communication with POMES will presumable take longer then the DMC timeout period and because the SNMP Proxy cannot send a response message to the DMC with out the DMC explicitly requesting it. - The correct DMC is the DMC that initially messaged the SNMP for that response. This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: Configure-1, Configure -1a, Configure -1b, General Failure-1, General Failure-2 |
| Basic Course |  | - DMC sends a SNMP “set” message (see Notes 1) containing the IP address of the device the that DMC user wishes to manage. - SNMP Proxy receives the SNMP “set” message - SNMP Proxy logs the IP address and Port of the DMC making the request (see Note 5) as well as the IP of the device the DMC user wishes to manage. - SNMP Proxy sends an SNMP response message to the DMC indicating “managed IP set”. - DMC receives the response message. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP boot object for the managed device that contains a boot mask (See Notes 2) - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC repeats SNMP request sent in Step 6 (via DMC User response to message sent in Step 11, see Notes 4) . - SNMP Proxy receives the repeated SNMP message. - SNMP Proxy sends an SNMP response to the DMC containing the corresponding response object that the DMC initially requested in Step 6. - DMC receives the requested response object. |
| Successful alternate course |  | For Step 5 A5.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A5.2 The DMC may repeat Step 1. (See Notes 6) For Step 15: A 15.1 SNMP Proxy receives the repeated SNMP message from the DMC but the SNMP Proxy has not received the required XML stream from the POMES (i.e. Step 13 did not occur). A 15.2 SNMP Proxy sends an SNMP message to the DMC indicating “waiting for server”. The DMC may repeat Step 14. |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy despite repeated attempts For Step 5: F 5.1 DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. For Step 8: F 8.1 SNMP Proxy connection to POEMS fails F 8.2 SNMP responds to DMC request with “server connection error” |
| Open Issues |  | Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? |

Configure Printer Trap Conditions and SNMP Alert

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Configure Printer Trap Conditions and SNMP Alert |
| Status |  | Under development, subject to design changes |
| Author |  | Team 6 |
| Purpose |  | To provide the ability to configure Trap conditions that cause devices to send out UDP based alert. |
| Overview |  | Users using an SNMP Management Console will be able to make entries into an trap table that represent printer device conditions for which a specified IP address should be notified of via a UDP Packet. The device conditions include toner and paper status as well as printer errors. The DMC first sends a message to the SNMP Proxy indicating which device it would like to manage. The DMC then sends a SNMP message containing an OID (see Note 1) of the SNMP trap entry object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the response object to the correct DMC (see Notes 5) using a stored IP Address. The use case will fail if either the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | Printronix Object Management Enterprise Server (POMES) |
| Precondition |  | - SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. - SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | An SNMP response object is received by the correct DMC (see Notes 5) from the SNMP Proxy that indicates that the boot protocal configuration was successful was successful. |
|  | Failure | No response received by the DMC that requested the information from the SNMP Proxy within the timeout period or an error has occurred (i.e. as a result of an invalid OID) that was reported and received by the correct DMC (see Note 5) from the SNMP Proxy that indicates the reason for the failure. |
| Related Use Cases |  | - Manage SNMP Access Control |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - See XMLCommands.doc (not included in this document) - The POMES server maintains the Community name for the SNMP proxy to use. - A User is expected to instruct the DMC to repeat an SNMP request upon receiving a “waiting for server” message. This is necessary because communication with POMES will presumable take longer then the DMC timeout period and because the SNMP Proxy cannot send a response message to the DMC with out the DMC explicitly requesting it. - The correct DMC is the DMC that initially messaged the SNMP for that response. This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: SNMP Alert-1, SNMP Alert -1a, SNMP Alert -1b, General Failure-1, General Failure-2 |
| Basic Course |  | - DMC sends a SNMP “set” message (see Notes 1) containing the IP address of the device the that DMC user wishes to manage. - SNMP Proxy receives the SNMP “set” message - SNMP Proxy logs the IP address and Port of the DMC making the request (see Note 5) as well as the IP of the device the DMC user wishes to manage. - SNMP Proxy sends an SNMP response message to the DMC indicating “managed IP set”. - DMC receives the response message. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP trap server object for the managed device that contains an IP address of an SNMP trap server. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP trap mask object for the managed device that will provide the device with a list of conditions to send traps for. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. |
| Successful alternate course |  | For Step 5 A5.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A5.2 The DMC may repeat Step 1. (See Notes 6) For Step 15: A 15.1 SNMP Proxy receives the repeated SNMP message from the DMC but the SNMP Proxy has not received the required XML stream from the POMES (i.e. Step 13 did not occur). A 15.2 SNMP Proxy sends an SNMP message to the DMC indicating “waiting for server”. The DMC may repeat Step 14. |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy despite repeated attempts For Step 5: F 5.1 DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. For Step 8 (or 16): F 8.1 SNMP Proxy connection to POEMS fails F 8.2 SNMP responds to DMC request with “server connection error” |
| Open Issues |  | Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? |

Configure Printer Trap Conditions and SMTP (Email) Alert

Use Case Diagram

Use Case Description

| Use Case Item |  | Description |
| --- | --- | --- |
| Name |  | Configure Printer Trap Conditions and SMTP (Email) Alert |
| Status |  | Under development, subject to design changes |
| Author |  | Team 6 |
| Purpose |  | To provide the ability to configure Trap conditions that cause devices to send out email based alert. |
| Overview |  | Users using an SNMP Management Console will be able to make entries into an trap table that represent printer device conditions for which a message should be sent to IP addresses via UDP. The device conditions include toner and paper status as well as printer errors. The DMC first sends a message to the SNMP Proxy indicating which device it would like to manage. The DMC then sends a SNMP message containing an OID (see Note 1) of the SMTP trap entry object which is received by the SNMP Proxy. SNMP Proxy replies with an SNMP message containing the response object to the correct DMC (see Notes 5) using a stored IP Address. The use case will fail if either the SNMP Proxy, DMC, or POMES do not receive sent messages. However, the DMC may repeat its request in the event of a timeout. |
| Priority |  | High |
| Actors | Primary | Device Management Console (DMC) |
|  | Secondary | Printronix Object Management Enterprise Server (POMES) |
| Precondition |  | - SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. - SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy - DMC setup to use SNMP Proxy MIB tree. |
| Post condition | Success | An SNMP response object is received by the correct DMC (see Notes 5) from the SNMP Proxy that indicates that the command was successful. |
|  | Failure | No response received by the correct DMC from the SNMP Proxy within the timeout period or an error has occurred either at the SNMP Proxy or the POMES (i.e as a result of an invalid OID). |
| Related Use Cases |  | N/A |
| Cross Reference & Notes |  | - See proxy.mib (not included in this document) - See XMLCommands.doc (not included in this document) - The POMES server maintains the Community name for the SNMP proxy to use. - A User is expected to instruct the DMC to repeat an SNMP request upon receiving a “waiting for server” message. This is necessary because communication with POMES will presumable take longer then the DMC timeout period and because the SNMP Proxy cannot send a response message to the DMC with out the DMC explicitly requesting it. - The correct DMC is the DMC that initially messaged the SNMP for that response. This is an important subtlety because multiple DMCs may be message the SNMP Proxy simultaneously. - SNMP uses UDP which makes no connection guarantees so DMC will not retransmit messages that do reach the SNMP Proxy. However the DMC may transmit new messages in response to the request of a DMC user (see Environmental Characteristics). - See Test Cases: SMTP Alert-1, SMTP Alert -1a, SMTP Alert -1b, General Failure-1, General Failure-2 |
| Basic Course |  | - DMC sends a SNMP “set” message (see Notes 1) containing the IP address of the device the that DMC user wishes to manage. - SNMP Proxy receives the SNMP “set” message - SNMP Proxy logs the IP address and Port of the DMC making the request (see Note 5) as well as the IP of the device the DMC user wishes to manage. - SNMP Proxy sends an SNMP response message to the DMC indicating “managed IP set”. - DMC receives the response message. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP SMTP server object for the managed device that contains an IP address of an SMTP server. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the email address object for the managed device that will provide the device with the email address to send trap messages to. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. - DMC sends a SNMP “set” message using one of the OIDs (see Notes 1) that corresponds to the SNMP trap mask object for the managed device that will provide the device with a list of conditions to send traps for. - SNMP Proxy receives the SNMP message. - SNMP Proxy sends an XML message stream corresponding to the SNMP message sent in Step 6 containing device data OID request (see Notes 2) to the POMES. - POMES receives the XML stream containing the device data OID. - SNMP Proxy sends an SNMP response message to the DMC indicating “waiting for server”. - DMC receives the “waiting for server message”. - POMES sends an XML stream containing a response object to the SNMP Proxy. - SNMP Proxy receives and stores a XML stream from the POMES containing the response object. |
| Successful alternate course |  | For Step 5 A5.1 The DMC does not receive a response from the SNMP Proxy within the timeout period. A5.2 The DMC may repeat Step 1. (See Notes 6) For Step 15: A 15.1 SNMP Proxy receives the repeated SNMP message from the DMC but the SNMP Proxy has not received the required XML stream from the POMES (i.e. Step 13 did not occur). A 15.2 SNMP Proxy sends an SNMP message to the DMC indicating “waiting for server”. The DMC may repeat Step 14. |
| Unsuccessful for all course |  | For Step 2: F 2.1 SNMP Messages transmitted by the DMC are not received by the SNMP Proxy despite repeated attempts. For Step 5: F 5.1 DMC does not receive any response from the SNMP Proxy within the timeout period despite repeated requests. For Step 8 (or 16 or 24): F 8.1 SNMP Proxy connection to POEMS fails F 8.2 SNMP responds to DMC request with “server connection error” |
| Open Issues |  | Are the timeout periods unique to each DMC or are they specified by the SNMP protocol? |

Non-Functional Constraints

Efficiency

- The SNMP Management console must receive a reply from the SNMP Proxy to SNMP requests within the timeout period (to be specified later in the range of one second)

- The SNMP Proxy must be able to efficiently handle simultaneous connections to an unspecified number of SNMP Management Consoles

Process

- Implementation  of the Proxy must be done in the Java using J2SE 1.4

- Implementation of the Proxy must conform to the Javadoc standard

External

- SNMP Proxy must be able to process requests from any standard SNMP Management Console using SNMP V1

- Any outside source code libraries used for implementation must be open source (preferable LGPL)

Lifecycle Considerations

After careful consideration and because of the makeup of the course, the lifecycle model that we will be using is the iterative and incremental lifecycle.  This lifecycle model will allow us to produce a specific deliverable of the project that can then be refined on a successive iteration.  The major deliverables produced by this lifecycle include:

- Requirements document

- Test Plan

- Design document

- Implementation

According to the course schedule, there will be two iterations of each deliverable of the project. A major benefit of using the iterative and incremental lifecycle model is that it allows for greater management of the system’s complexity since only a portion of the system is being worked on at a single time.  The lifecycle model also allows for greater customer feedback since incremental releases are made available to the client.  The incremental releases can also be used for internal testing, evaluation, and milestones.

Acceptance Requirements

The SNMP Proxy must be able to provide device management console users with the functionality described in the Use Case section.  Use cases will be considered acceptable when they are successfully tested using the specified test plans (i.e. when all inputs match expected outputs).

Unit Test Plan

| ### Test Case Id | Unit-1 |
| --- | --- |
| ### Unit Being Tested | UDPComManager |
| ### Test Case Description | Listen() - The UDPComManager listens on the designated port for reception of a UDP packet. Every command that a DMC issues (i.e. setManagedIP, getGeneralInfo, etc…) must be received in order to be processed |
| ### Input | UDP packet from the Device Management Console. |
| ### Expected Output | An SMTP packet containing the DMC session info and command |

| ### Test Case Id | Unit-2 |
| --- | --- |
| ### Unit Being Tested | UDPComManager |
| ### Test Case Description | sendUDPPacket() – sends a UDP packet to a designated DMC |
| ### Input | An SNMP packet containing the response expected from the DMC |
| ### Expected Output | The UDP packet designated to a DMC |

| ### Test Case Id | Unit-3 |
| --- | --- |
| ### Unit Being Tested | SNMPProcessor |
| ### Test Case Description | genSNMPPacket() – parses an SNMP packet |
| ### Input | An SNMP packet |
| ### Expected Output | The command contained in the SNMP packet and session id |

| ### Test Case Id | Unit-4 |
| --- | --- |
| ### Unit Being Tested | SNMPProcessor |
| ### Test Case Description | genSNMPResponse() – based on the SNMP command received, the proper response is generated |
| ### Input | the SNMP command |
| ### Expected Output | The corresponding response in relation to the SNMP command received |

| ### Test Case Id | Unit-5 |
| --- | --- |
| ### Unit Being Tested | SNMPProcessor |
| ### Test Case Description | genSNMPWait() – a DMC must be notified that the POMES is processing its command |
| ### Input | (none) |
| ### Expected Output | An SNMP packet with containing a waiting notification |

| ### Test Case Id | Unit-6 |
| --- | --- |
| ### Unit Being Tested | ProtocolManager |
| ### Test Case Description | haveReceivedUDP() – the ProtocolManager must be notified that an SNMP command (wrapped in a UDP packet) was issued to the proxy |
| ### Input | SNMP packet containing DMC information and command |
| ### Expected Output | the SNMP packet to be parsed by the SNMPProcessor |

| ### Test Case Id | Unit-7 |
| --- | --- |
| ### Unit Being Tested | ProtocolManager |
| ### Test Case Description | haveReceivedXML() – the ProtocolManager must be notified that a response from the POMES has been received |
| ### Input | An XML string |
| ### Expected Output | An XML string to be parsed |

| ### Test Case Id | Unit-8 |
| --- | --- |
| ### Unit Being Tested | SessionTable |
| ### Test Case Description | createSession() – a DMC issuing commands and requests must be tracked via its IP address |
| ### Input | The DMC IP address |
| ### Expected Output | Confirmation that the IP address has been successfully recorded |

| ### Test Case Id | Unit-9 |
| --- | --- |
| ### Unit Being Tested | SessionTable |
| ### Test Case Description | lookUpSession() – a DMC that has already created a session will have its information already stored in the SessionTable |
| ### Input | The DMC IP address |
| ### Expected Output | The session information of an already existing DMC session |

| ### Test Case Id | Unit-10 |
| --- | --- |
| ### Unit Being Tested | SessionTable |
| ### Test Case Description | updateSession() – session information for the DMC must be updated to keep track of if the POMES has responded to a command |
| ### Input | The DMC IP address, session status |
| ### Expected Output | The session information of an already existing DMC session |

| ### Test Case Id | Unit-11 |
| --- | --- |
| ### Unit Being Tested | XMLProcessor |
| ### Test Case Description | genXMLStream() – based on the SNMP command received from a DMC, a corresponding XML command must be generated in order to communicate with the POMES |
| ### Input | Command that must be wrapped in an XML string |
| ### Expected Output | The XML string containing the command |

| ### Test Case Id | Unit-12 |
| --- | --- |
| ### Unit Being Tested | XMLComManager |
| ### Test Case Description | sendXMLStream() – A stream with POMES must be opened in order for POMES to receive XML commands from the proxy |
| ### Input | XML string corresponding to the SNMP packet sent by the DMC. |
| ### Expected Output | An XML stream containing a response object |

Integration Test Plan

| ##### Test Case ID | Request-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages sent from DMC to SNMP Proxy for use in the “Request General Proxy Information” use case result in receiving the correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console |
| Input Specification | DMC sends a SNMP “get” messages using an OID from the “general” MIB subtree |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the community name setting was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | Request-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages sent from DMC to SNMP Proxy for use in the “Request General Proxy Information” use case result in receiving the correct response object, even after the DMC’s timeout period |
| Items being tested | SNMP Proxy communication to SNMP Console |
| Input Specification | DMC sends a SNMP “get” messages using an OID from the “general” MIB subtree |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the community name setting was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | Manage-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP “get” messages sent from DMC to SNMP Proxy for use in the “Manage SNMP Access Control” use case result in receiving the correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP, SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “proxySNMP” MIB subtree corresponding to retrieving a community name. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the community name setting was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | Manage-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP “set” messages sent from DMC to SNMP Proxy for use in the “Manage SNMP Access Control” properly handles getting no response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP. SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “proxySNMP” MIB subtree corresponding to setting a community name. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the command was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | Manage-1b |
| --- | --- |
| ### Purpose | To ensure the SNMP “set” messages sent from DMC to SNMP Proxy for use in the “Manage SNMP Access Control” use case results in a response even prior to the command response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP. SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “proxySNMP” MIB subtree corresponding to setting a community name. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the command was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | Issue-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages sent from DMC to SNMP Proxy for use in the “Issue Device Commands” use case result in correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP. SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipCongfigure” MIB subtree corresponding to boot settings. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the command was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | Issue-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages sent from DMC to SNMP Proxy for use in the “Issue Device Commands” properly handles getting no response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP. SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipCongfigure” MIB subtree corresponding to boot settings. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the command was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | Issue-1b |
| --- | --- |
| ### Purpose | To ensure the SNMP messages sent from DMC to SNMP Proxy for use in the “Issue Device Commands” use case results in a response even prior to the command response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP. SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipCongfigure” MIB subtree corresponding to boot settings. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the command was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | Configure-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “get” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” use case result in receiving the correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | Configure-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “get” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” properly handles getting no response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | Configure-1b |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “get” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” use case results in a response even prior to the command response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | Configure-2 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “set” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” use case result in receiving the correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | Configure-2a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “set” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” properly handles getting no response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | Configure-2b |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to “set” a device’s boot protocol sent from DMC to SNMP Proxy for use in the “Configure Device Boot Settings” use case results in a response even prior to the command response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “commands” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the boot configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | SNMP Alert-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SNMP Alert” use case result in receiving the correct response object. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipTrapTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | SNMP Alert-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SNMP Alert” properly handles getting no response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipTrapTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | SNMP Alert-1b |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SNMP Alert” use case results in a response even prior to the command response from the server. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipTrapTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | SMTP Alert-1 |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SMTP Alert” use case result in receiving the correct response object. Testing this requires a valid STMP outgoing mail server and e-mail address. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipSmtpTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | SMTP Alert-1a |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SMTP Alert” properly handles getting no response from the server. Testing this requires a valid STMP outgoing mail server and e-mail address. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipSmtpTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server does not respond within proxy’s timeout period, and the DMC is forced to issue the command again. |

| ##### Test Case ID | SMTP Alert-1b |
| --- | --- |
| ### Purpose | To ensure the SNMP messages to make entries into a device’s SNMP trap table sent from DMC to SNMP Proxy for use in the “Configure Printer Trap Conditions for SMTP Alert” use case results in a response even prior to the command response from the server. Testing this requires a valid STMP outgoing mail server and e-mail address. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP SNMP Proxy communication to POMES via XML. |
| Input Specification | DMC sends a SNMP messages using an OID from the “tcpipSmtpTable” MIB subtree. |
| Output Specification | An SNMP response object is received by the correct DMC from the SNMP Proxy that indicates that the trap configuration was successful or that indicates an error has occurred and explaining its cause. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs as well as listening for XML streams from the POMES. SNMP Proxy has an authenticated TCP/IP connection with POMES which is listening for XML streams from SNMP Proxy DMC setup to use SNMP Proxy MIB tree. The server has not yet responded to the command issued by the DMC. |

| ##### Test Case ID | General Alternate-1 |
| --- | --- |
| ### Purpose | To test the SNMP Proxy response to failing to respond to a DMC message within the DMC timeout period. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP |
| Input Specification | DMC sends a SNMP messages to the SNMP Proxy however the SNMP replies to the messages by the SNMP Proxy fail to reach the DMC within timeout period (i.e. DMC configured to time out after 0.00001 seconds). |
| Output Specification | SNMP Proxy should listen for additional requests despite timeout. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs. DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | General Alternate-2 |
| --- | --- |
| ### Purpose | To test the SNMP Proxy response to receiving a repeated request from a DMC for device data when it has not yet received the required information from the POMES. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP |
| Input Specification | DMC repeatedly sends a SNMP messages to the SNMP Proxy requesting a device data OID. |
| Output Specification | SNMP Proxy should send an SNMP response to each message from DMC indicating “waiting for server”. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs. No XML reply from POEMS given to SNMP Proxy XML commands despite a successful connection. DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | General Failure-1 |
| --- | --- |
| ### Purpose | To test the SNMP Proxy response to failing to receive messages from a DMC during a use case. |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP |
| Input Specification | DMC sends a SNMP messages to the SNMP Proxy after which communication between DMC and SNMP Proxy terminated. |
| Output Specification | SNMP Proxy should timeout the session with the DMC and continue to listen for additional DMC messages. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs. DMC setup to use SNMP Proxy MIB tree. |

| ##### Test Case ID | General Failure-2 |
| --- | --- |
| ### Purpose | To test the SNMP Proxy response to receiving a request from a DMC for device data when communication to the POEMS has failed |
| Items being tested | SNMP Proxy communication to SNMP Console via SNMP |
| Input Specification | DMC sends a SNMP messages to the SNMP Proxy requesting a device data OID. |
| Output Specification | SNMP Proxy should send an SNMP response to each message from DMC indicating “server connection error”. |
| Test Environment | SNMP Proxy listening on specified port for SNMP UDP packets from DMCs. Communication between the POMES and SNMP Proxy intentionally interrupted. DMC setup to use SNMP Proxy MIB tree. |

System Test Plan

| ##### Test Case ID | System-1 |
| --- | --- |
| ### Description | Retrieve general information |
| ### Purpose | Verify the product information and manufacturer information of the proxy agent |
| Input Specification | DMC sends a command requesting specific information concerning the proxy or manufacturer of the proxy |
| Output Specification | SNMP sends to the DMC the information that was requested |

| ##### Test Case ID | System-2 |
| --- | --- |
| ### Description | Reset the manage device |
| ### Purpose | Verify that the DMC’s managed device resets |
| Input Specification | DMC sends a reset command to an managed device |
| Output Specification | The specified device resets and sends a message that the command was successful to the DMC |

| ##### Test Case ID | System-3 |
| --- | --- |
| ### Description | Set and retrieve community name |
| ### Purpose | Verify the functionality of the community information |
| Input Specification | DMC sets and gets the proxy community information of the managed device |
| Output Specification | Server sends to the DMC the community information of its managed device |

| ##### Test Case ID | System-4 |
| --- | --- |
| ### Description | Set the printer to send out SNMP traps |
| ### Purpose | Verify that the DMC receives changes notifications from the managed device |
| Input Specification | DMC sets the destination trap server address and mask |
| Output Specification | Server sends to the DMC, depending on the mask set, the trap message is sent when the trap event occurs specified by the mask. |

| ##### Test Case ID | System-5 |
| --- | --- |
| ### Description | Set the printer to send out email alerts |
| ### Purpose | Verify that the emails are sent when an event occurs for the device |
| Input Specification | DMC sets the SMTP IP address, email address to send notifications, and mask |
| Output Specification | Server sends via the SMTP IP address, a notification to the email address when the trap event occurs specified by the mask. |

Glossary

- DMC –Device Management Console.  These are client applications that are used fore managing network devices via SNMP.

- Java™ - A highly portable, general-purpose programming language developed by Sun Microsystems in the early- to mid-1990’s.

- MIB – A set of managed objects referred to as the SNMP Management Information Base representing the information available via SNMP.

- OID – Object Identification.  This identifies a specific manageable object within a MIB.

- Open Source – A licensing standard that allows programmers to read, redistribute, and modify the source code for a piece of software

- POMES – Printronix Output Management Enterprise Server.  This server manages the Printronix printers and communicates with the SNMP Proxy via XML.

- Proxy – A server that acts as an intermediary between a workstation user and a network so that security, and/or control services can be provided.

- SNMP – An application layer protocol used to monitor and configure a simple TCP/IP network.

- UDP – the User Datagram Protocol is a connectionless service. This results in a low overhead and fast transfer service (relies on the upper layer protocols to provide error checking and delivery of data).

- XML – Short for Extensible Markup Language, a specification that allows designers to create their own customized tags, enabling the definition, transmission, validation, and interpretation of data between applications and between organizations.and between organizations.